07th March 2018

From: Peter Bos, (on behalf of Cycle Action Waikato) approved by members, meeting 06/03/18

cawaikato@can.org.nz

To: Evan Freshwater Evan.Freshwater@saferoads.co.nz

And Erik van der Wel Erik.vanderWel@saferoads.co.nz

Safe Roads Alliance Project SH23 Hamilton to Whatawhata to Raglan

Thanks for your time on Friday 2nd March 2018 to meet with me.

First words:

For weaker road users the proposed design is Dangerous by design. A safe system approach accepts people make mistakes. The proposed mistake allowance is below zero in many places.

Main points:

- CAW expects the slowing of traffic by design to a safe speed and the providing pedestrian islands where children cross

- Narrower cycle lanes [are] three to four times less safe than wider cycle lanes

- CAW asks the designer of road to recommend speed review in writing to Elected officials for SH23 & roads local to Te Araroa route West of Hamilton. WDC & Cc HCC

- CAW Supports NZTA building 3m wide shared-use path separated from high speed traffic, along SH23 from Hamilton past/to Whatawhata/Raglan

Feed back with references:

- OF MOST SERIOUS CONCERN – CHILDREN CROSSINGS

Where we know children are crossing the road (SH23), we all have a moral duty to take an uncompromising Vision Zero approach.

'Life and health [of children] can never be exchanged for other benefits within the society'

CAW expects the slowing of traffic by design to a safe speed and the providing pedestrian islands where children cross

Guiding Reference: Vision Zero NYC (slide22) - The Science of Safe Streets

- New crosswalks where people want to cross

- Shorter crossing distances pedestrian islands

- Clearer lane marking

- SERIOUS CONCERN – SHOULDER WIDTH

“Narrower cycle lanes [are] three to four times less safe than wider cycle lanes.” (page 9) NZTA report 389 Cycle Safety: Reducing the Crash Risk: October 2009

Guiding references

“A one-way cycle-track of 2.00 m or narrower is not a good cycling-facility”

CROW Design manual for a cycle-friendly infrastructure 1996 (Table 4.3*)

Copenhagen guidelines for the design of road projects – Focus on cycling

Chapter 2 Cycle tracks: Minimum width of a Copenhagen cycle track is 2.2 m

On individual sections, where there is only enough space for a very narrow cycle track (1.7-1.8 m), the cycle track may be installed if planners decide that cyclist safety, security and pass-ability taken as a whole would be improved in relation to the current situation.

End page 1 of 2

- SERIOUS CONCERN – SPEED, SHARED RESPONSIBILITY & SAFE SYSTEM

- Our bodies have a limited ability to withstand crash forces without being seriously injured or killed.

- Those who design the road system ... share responsibility for creating a road system where crash forces don't result in death or serious injury.

“The optimal speed limit on an urban street is the lowest limit the political decision makers can accept” http://www.trafikdage.dk/td/papers/papers04/Trafikdage-2004-339.pdf

CAW asks the designer of road to recommend speed review in writing to Elected officials for SH23 & road local to Te Araroa route West of Hamilton. WDC & Cc HCC

- IN SUPPORT

When People go driving, biking, walking with a friend they will always what to be side by side, this is normal for all modes of traffic.

CAW Supports NZTA building 3m wide shared-use path separated from high speed traffic, along SH23 from Hamilton past/to Whatawhata/Raglan

CAW expects off-road path widths to be designed for social walking and biking, like driver get.

Last words:

The Hamilton - Whatawhata - Raglan is an irresistible ride and destination. But speed & behaviour of motorist is scary, making it a once in a life time ride. Build it safe and more will ride, multiplying safety with safety in numbers.

End.